1. Core Networking Concepts

1. What is a Network Switch?

A Switch is a Layer 2 (Data Link Layer) device used to connect multiple devices (PCs, Servers, Printers) within a Local Area Network (LAN).

- How it works: Unlike a Hub, a switch is "intelligent." It learns the MAC Addresses of connected devices and stores them in a MAC Address Table.

- Key Benefit: It sends data only to the specific port where the destination device is connected, which reduces network traffic and prevents data collisions.

2. Switch Boot Sequence

When you power on a Cisco switch, it follows these steps:

- POST (Power-On Self-Test): The switch tests its hardware (CPU, RAM, and Flash).

- Run Boot Loader: A small program in ROM starts the boot process.

- Low-level CPU Initialization: Prepares the CPU for operation.

- Initialize Flash File System: Accesses the storage where the OS is kept.

- Load IOS: The Operating System (IOS) is moved from Flash to RAM.

- Load Configuration: The startup-config file is loaded into the running-config.

3. Three Methods to Access Switch CLI

The Command Line Interface (CLI) can be accessed via:

- Console Port: A physical connection using a console cable. This is used for initial setup or when the network is down.

- Telnet: A remote access method over the network. It is insecure because data is sent in plain text.

- SSH (Secure Shell): A secure remote access method that encrypts all data, making it the preferred choice for administrators.

4. Cisco IOS (Internet Operating System)

The IOS is the software that manages Cisco devices. It operates in different modes:

- User EXEC Mode (Switch>): Limited "view-only" mode.

- Privileged EXEC Mode (Switch#): Allows you to view detailed configurations (e.g., show run).

- Global Configuration Mode (Switch(config)#): Used to make actual changes to the device (e.g., setting a hostname).

5. Switch Port Types

- Access Port: Used for end devices (like a computer). It belongs to only one specific VLAN.

- Trunk Port: Used to connect two switches. It can carry traffic for multiple VLANs simultaneously.

2. Assignment Solutions

Question: Router Neighbor Relationship (R1-R4)

Note: Since the exhibit (image) is missing, the rule for OSPF/Routing neighbors is that Subnet Masks, Area IDs, and Hello/Dead Timers must match. If R1 and R2 have matching configurations, they will form the relationship.

Question 3: enable secret hashing algorithm

Answer: A. MD5

Reason: In Cisco IOS, the enable password command uses plain text, but the enable secret command uses the MD5 hashing algorithm by default to protect the password.

Question 4: OSPF Neighbor status FULL/BDR

Answer: D. Router 2.2.2.2 is a backup designated router.

Reason: When you see FULL/BDR in the neighbor table, the first part (FULL) is the state, and the second part (BDR) refers to the role of the neighbor (the router at 2.2.2.2).

Question 5: Command to view neighbor discovery table on a PC

Answer: D. netsh interface ipv6 show neighbors

Reason: This is the specific Windows command-line utility used to view the IPv6 neighbor cache (similar to the ARP table in IPv4).

Question 6: Variable type Routers = [R1, R2, R3]

Answer: A. List

Reason: In programming (specifically Python), any collection of items enclosed in square brackets [] is defined as a List.

Question 7: Fields in an IPv4 header (Choose three)

Answer: B. Time to Live, C. Source address, D. Destination address

Reason: TTL, Source IP, and Destination IP are actual fields within the IPv4 packet header. "Network" and "Host" are parts of an IP address, not header fields.